# Technical Evaluation of Roofing Systems and Structural Integrity for the FAMAD PLC Industrial Facility in Ojota, Lagos

## Industrial Heritage and Strategic Positioning of the FAMAD Facility

The industrial evolution of Nigeria is intrinsically linked to the development of the Ikorodu Road corridor in Lagos, a primary artery that has facilitated the movement of goods and labor for over six decades. FAMAD PLC, formerly operating under the globally recognized Bata brand, established its footprint at Kilometer 15, Ikorodu Road, Ojota, in 1960. The FAMAD facility remains a cornerstone of this heritage, producing a diverse range of footwear, including industrial work boots, school shoes, and sneakers tailored for the Nigerian environment.

The site at Kilometer 15 is a substantial industrial holding, with a total fenced land area of 56,819.28 square meters. A detailed analysis of the aerial imagery reveals a complex arrangement of high-bay warehouses and specialized utility zones. The facility's location is strategically vital, situated between the high-traffic Ikorodu Road to the East and a dense residential/commercial mix to the West. This proximity to critical infrastructure like the Lagos Metropolitan Area Transport Authority (LAMATA) Quality Bus Corridors (QBC) project introduces unique environmental stressors that directly impact the longevity of the facility’s roofing systems.

Industrial operations of this scale require robust roofing infrastructure to protect sensitive raw materials, such as leather and adhesives, as well as high-precision manufacturing equipment. The roofing system at FAMAD PLC is not merely a weather barrier but a critical component of the climate control and safety architecture of the plant. Given the facility’s age, the roofing materials have transitioned through multiple maintenance cycles, necessitating a comprehensive evaluation of their current structural state and their resilience against the aggressive atmospheric conditions of the Ojota region.

## Atmospheric Corrosion and Environmental Stressor Analysis

The geographical location of the FAMAD PLC factory in Ojota places it within one of the most chemically aggressive environments in the Lagos metropolis. The primary driver of this hostility is the facility’s proximity to the Olusosun landfill, a sprawling waste disposal site covering between 42 and 100 acres.10 Since its inception in 1989, the landfill has become a major source of atmospheric pollutants, including sulfur dioxide (), nitrogen oxides (), and hydrogen sulfide (), resulting from the decomposition of organic waste and the frequent burning of electronic waste.

The interaction between these pollutants and the high relative humidity of Lagos (often exceeding 80% year-round) creates a corrosive aerosol that settles on the expansive roof surfaces of the FAMAD plant. 

Elevation Estimate

| Level | Height |
| --- | --- |
| Minimum | 19.9 m |
| Median | 22.23m |
| Maximum | 24.55m |

| Property Attribute | Technical Detail |
| --- | --- |
| Facility Name | FAMAD PLC (Formerly Bata Nigeria) [1] |
| Primary Address | Kilometer 15, Ikorodu Road, Ojota, Lagos, Nigeria |
| Year Established | 1960 |
| Total Confirmed Land Area | 56,819.28 square meters |
| Total Site Perimeter | 967.12 meter |
| Primary Production Focus | Industrial, School, and Casual Footwear [2] |
| Surrounding Infrastructure | Ikorodu Road, LAMATA BRT Corridor, Olusosun Landfill |

The FAMAD PLC facility is a vital part of the Ojota industrial landscape. By utilizing advanced aerial diagnostics and adhering to the confirmed 93m structural reference, the facility can mitigate the aggressive environmental risks posed by its proximity to the Olusosun landfill and Ikorodu Road. This data-driven approach ensures that the "FAMAD family" remains protected under a roof as durable as the footwear they produce.



FAMAD FACTORY

Roof Area Calculation Summary

1. Big Roof Specification

Roof Pitch: 18°

One Slab Dimensions

| Parameter | Value |
| --- | --- |
| Width | 8.83 m |
| Length | 93 m |

Area per slab

8.83 × 93
 = 821.19 m²

Number of segments: 13

Total slab area:

821.19 × 13
 = 10,675.47 m²

2. Additional Roof Segments

Front Segment

Dimensions:

10.52 m × 93 m

Area:

= 978.36 m²

Back Segment

Dimensions:

7.68 m × 93 m

Area:

= 714.24 m²

Roof Flaps

Number of flaps: 15

Area per flap: 100 m²

Total flap area:

100 × 15
 = 1,500 m²

3. Total Roof Area

| Component | Area |
| --- | --- |
| Main roof slabs | 10,675.47 m² |
| Front segment | 978.36 m² |
| Back segment | 714.24 m² |
| Roof flaps | 1,500 m² |

Total Roof Area

10,675.47 + 978.36 + 714.24 + 1,500

= 13,868.07 m²

| LABEL | STRUCTURE | PERIMETER(m) | TOTAL AREA(m²) | ROOF COVERAGE(m²) |
| --- | --- | --- | --- | --- |
| A | KINSTAR/LONTOR OFFICES | 186.66 m | 1,243.54 | 1189.7 |
| B | FAMAD CENTRAL STORE | 80.759 | 387.65 | 373.6 |
| C | GAC MOTORS | 49.036 | 145.51 | 137.8 |
| D | FAMAD ADMIN OFFICE | 148.938 | 1,003.52 | 898.4 |
| E | FAMAD FACTORY | 464.581 | 12,963.32 | 13,868.07 |
| F | GAC WAREHOUSE | 144.612 | 940.14 | 960.4 |
| G | KINLIFE | 187.957 | 1,155.43 | 465.9 |
| H | ELIZABETH R/GATIMO | 298.993 | 3,656.86 | 3705.1 |
| I | GENERATOR ROOM | 78.32 | 294.94 | 329.6 |
| J | MTN-J | 131.731 | 936.4 | 922.1 |
| K | MTN-K | 185.736 | 1,877.01 | 1,442.70 |
| L | MTN-L | 107.559 m | 644.21 | 623.3 |
| M | MTN-M | 78.129 | 316.48 | 315.6 |
| N | MTN-N | 130.413 | 978.2 | 962.3 |
| O | MTN-O | 178.559 | 1,447.27 | 1285.1 |
| P | MTN-P | 154.479 | 971.1 | 749.2 |
| Q | MTN-Q | 73.251 | 282.61 | 278.4 |
| R | MTN-R | 53.242 | 176.66 | 176.3 |
| S | MTN-S | 63.027 | 248.32 | 248.5 |
| T | MTN-T | 54.613 | 179.53 | 178 |